- Apply 3's rule to estimate what 16 mWs is in dBm. _____

- Apply 3's rule to estimate what 0.0625 mWs is in dBm. _____

- Apply 10's rule to estimate what 10,000 mWs is in dBm. _____

- Apply 10's rule to estimate what 0.0001 mWs is in dBm. _____

- Apply 3's rule to estimate what 15 dBm is in mW. _____

- Apply 3's rule to estimate what -15 dBm is in mW. _____

- Apply 10's rule to estimate what 50 dBm is in mW. _____

- Apply 10's rule to estimate what -50 dBm is in mW. _____

#### Task 2: Estimating power levels using 3's and 10's rules

In cascaded systems with power gain and/or loss along a chain of subsystems, power levels given in dB forms can be directly added or subtracted to calculate the total power.

For instance, a transmitter with a power of 9 dBm, after a cable and connector loss of 3 dB, ends up with a power of 9 dBm ˗ 3 dB = 6 dBm or 4 mW. Notice that the simple addition and subtraction here don't apply to powers given in mW.

If one were to subtract 3 dB from 8 mW (i.e., mixed measurement units), 8 mW should be converted to 9 dBm first. The result is in dBm, which can be converted to mW when needed.

- Applying the 3's and 10's rules, what's 0.125 mW ˗ 30 dBm in dBm? _____

- Applying the 3's and 10's rules, what's 0.125 mW ˗ 30 dBm in mW? _____

- Applying the 3's and 10's rules, what's 0.0625 mW ˗ 40 dBm in dBm? _____

- Applying the 3's and 10's rules, what's 0.0625 mW ˗ 40 dBm in mW? _____

- Applying the 3's and 10's rules, what's 8 mW + 30 dBm in dBm? _____

- Applying the 3's and 10's rules, what's 8 mW + 30 dBm in mW? _____

- Applying the 3's and 10's rules, what's 16 mW + 40 dBm in dBm? _____

- Applying the 3's and 10's rules, what's 16 mW + 40 dBm in mW? _____

The same idea can be applied to conversions from dBm to mW when a dBm value is not readily available in the 3's and 10's rules table.

For instance, what is 13 dBm in mW?

First, 13 dBm = 0 dBm + 10 dBm + 3 dBm, where 0 dBm, 10 dBm, and 3 dBm are in the table.

Next, let's apply the following 3's and 10's rules:

- 0 dBm = 1 mW

- When the dBm value increases by adding 10, the mW value increases by the factor of 10.

- When the dBm value increases by adding 3, the mW value increases by the factor of 2.

Finally, we have 13 dBm = 0 dBm + 10 dBm + 3 dBm = 1 mW × 10 × 2 = 20 mW.

- Applying the 3's and 10's rules, what's 7 dBm in mW (must show steps)?

Answer:

- Applying the 3's and 10's rules, what's 4 dBm in mW (must show steps)?

Answer:

- Applying the 3's and 10's rules, what's 9 dBm in mW (must show steps)?

Answer:

- Applying the 3's and 10's rules, what's -7 dBm in mW (must show steps)?

Answer: