Complete the following tasks using the Toolwire® lab, "Build Your Own Lab in a 4 Router WAN Topology":

- Determine and apply the basic configuration on all devices

- Determine and apply the enhance security on switch S1 to meet the following requirements:

  - Enable port security on the S1 interface to allow only the PC to connect to the switch

  - Violation action: Protect

  - Max MAC addresses: 1

- Configure and activate the interfaces on the devices with the IP addresses from your IP Address Plan

- Configure the PC workstation IP address, subnet mask, and gateway

- Add VLANs and trucking as necessary to S1 to allow VLANs 1, 10, and 20 to your configuration from Week Two

6.    Use Point-to-Point Protocol (PPP) on the WAN link connecting Core and R4 routers
7.    Use frame-relay to connect R1 and Core routers

- Configure a point-to-point subinterface WAN link from R1 to R2; DLCI = 200

- Configure a point-to-point subinterface WAN link from Core to R2; DLCI = 200

8.    Router R2 will be used as our frame switch

- Type the following configuration from global configuration mode on R2:

frame switch
!
interface Serial0/0/0
 no ip address
 encapsulation frame-relay
 frame-relay intf-type dce
 frame-relay route 200 interface Serial0/0/1 200
!
interface Serial0/0/1
 no ip address
 encapsulation frame-relay
 frame-relay intf-type dce
 frame-relay route 200 interface Serial0/0/0 200

9.    Save your configurations on the devices

10..    Save your configurations in Toolwire®

You should be able to ping all directly connected devices, verify your configurations:

- From Core, ping 172.16.255.254

- From Core, ping 192.168.1.241

- From R4, ping 172.16.0.1

- From R1, ping 192.168.1.242

- Create an OSPF routing process 1 for area 0 on R1 and all interfaces on Core except S0/1/0

- Create an OSPF routing process 1 for area 1 on R4 and Core S0/1/0

- Save your configurations on the devices

- Save your configurations in Toolwire®

You should be able to ping all devices from any network location; verify your configurations:

- Ping the R4 Loopback0 interface 172.17.0.1 from PC successfully

- Ping PC IP address from R4 router successfully

- Assign 209.165.200.224/30 network IP addresses to the interfaces connecting Core and R4 routers

- On the Core router, configure NAT with overload (PAT) to translate any 192.168.1.0/24 IP address to the IP address configured to the serial interface connecting to R4 router

- Configure a default route on the Core router to R4 and propagate the route in OSPF

- Configure a static route on the R4 router to reach 192.168.1.0/24 networks