SFTY 355 Industrial Hygiene and Toxicology

HOMEWORK #6

Name _ Assignment Grade: 100%__________________________________

- (24 pts) When workers are at risk for cold stress, describe at least 3 general and three specific controls that should be implemented

General Controls:

- Training

- Hygiene Practices

- Medical Surveillance

Specific Controls:

- Warming Shelters

- Work/Rest Cycles

- Wind Barriers

- (8 pts) What are two physiological responses to cold stress?

- Conservation of body heat through reduced blood circulation

- Shivering to increase the metabolic rate

- (16 pts) Describe at least 4 work practice controls that may be used for working in a hot environment.

- Shorten duration of each exposure

- Schedule hot jobs during the cooler part of the day

- Self-limitation

- Air conditioned recovery area

- (12 pts) What are the three main characteristics of clothing that affect thermal balance?

- Insulation

- Permeability

- Ventilation

- Complete the table (40 pts)

| Globe temp (F) | 94 | 89 | X | X |
| --- | --- | --- | --- | --- |
| Dry bulb temp (F) | 91 | X | 98 | 94 |
| Natural wet bulb temp (F) | 74 | 86 | X | X |
| Botsball (F) | X | X | 87 | 92 |
| WBGT (F) | 80 | 86.9 | 90.5 | 93.7 |
| WBGT (C) | 26.7 | 30.5 | 32.5 | 34.3 |
| Approximate ACGIH work/rest regimen for acclimatized worker performing light work | Cont. | 75/25 | X | X |
| Approximate ACGIH work/rest regimen for un-acclimatized worker performing moderate work | 20/80** | X | X | X |

**I was unable to find specific information on un-acclimatized workers, so I went with the recommendation of the acclimatization process for a new worker on the first day which is 20% work.