LAB RESERVATION ID XXXXXXX

PART 2

Step 2: Configure the Router

- Console into the router and enable privileged EXEC mode :

Router>enable

(b)Enter configuration mode:

Router#configure terminal

(c) Assign a device name to the router:

Router(config)# hostname R1

(d) Disable DNS lookup to prevent the router from attempting to translate incorrectly entered commands as though they were host names :

R1(config)# no ip domain-lookup

(e)Assign class as the privileged EXEC encrypted password

R1(config)# enable secret class

(f)Assign cisco as the console password and enable login

R1(config)# line con 0

R1(config-line)#password cisco

R1(config-line)#login

R1(config-line)#exit

(g)Assign cisco as the VTY password and enable login

R1(config)#line vty 0 4

R1(config-line)#password cisco

R1(config-line)#login

R1(config-line)#exit

R1(config)#

(h)Encrypt the clear text passwords

R1(config-line)#service password-encryption

(i)Create a banner that warns anyone accessing the device that unauthorized access is prohibited

R1(config)#banner motd#Unauthorized access prohibited#

(j & k)Configure and activate both interfaces on the router and configure description for each interface indicating which device is connected to it

R1(config)# int g0/0

R1(config-if)# description Connection to PC-B

R1(config-if)# ip address 192.168.0.1 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)# int g0/1

R1(config-if)# description 192.168.1.1 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#exit

R1#

(l)Save the running configuration to the startup configuration file

R1# copy running-config startup-config

(m)Set the clock on the router

R1# clock set 11:00:00 April 1 2016

PART 3:

Step 1:

(a)Use the show version command to answer the following questions about the router

R1# show version

(b)Use the show version command to answer the following questions about the switch

Switch# show version

Step 2:

Use the show ip route command on the router to answer the following questions

R1# show ip route

Step 3:

Use the show interface g0/1 to answer the following questions

R1# show interface g0/1

Step 4:

- Enter the show ip interface brief command on the router

R1# show ip interface brief

(b)Enter the show ip interface brief command on the switch

Switch# show ip interface brief

Initialize the Router and Reload

R1#erase startup-config

R1#reload

no

no

yes

Initialize the Switch and Reload

switch>enable

switch# show flash

switch#delete plan.dat

switch#erase startup-config

switch@reload

no

no

ANSWERS TO QUESTIONS

Step 1(c)

Ping PC-B from a command prompt window on PC-A.
Why were the pings not successful?

MY ANSWER:

Because the router interfaces (default gateways) have not been configured yet.

Step 2(n)

Ping PC-B from a command prompt window on PC-A.

Were the pings successfully? Why?

MY ANSWER:

Yes, the pings were successful. The router has been configured and can route the traffic across the two subnets.

PART 3:

Step 1(a)

What is the name of the IOS image that the router is running?

ANSWER:

C1900-UNIVERSALK9-MZ.spa.152-4.M3.bin

How much DRAM memory does the router have?

ANSWER:

512MB

How much NVRAM memory does the router have?

ANSWER:

255K bytes of non-volatile configuration memory

How much Flash memory does the router have?

ANSWER:

250880K bytes of ATA System CompactFlash 0(Read/Write)

Step 1(b)

What is the name of the IOS image that the switch is running?

ANSWER:

c2960-lanbasek9-mz.150-2.SE.bin

How much dynamic random access (DRAM) does the switch have?

ANSWER:

65536 of memory

How much nonvolatile random access memory (NVRAM) does the switch have?

ANSWER:

64K bytes

What is the model number of the switch?

ANSWER:

WS-C2960-24TT-L

Step 2:

What code is used in the routing table to indicate a directly connected network?

ANSWER:

C

How many route entries are coded with a C code in the routing table?

ANSWER:

2

What interface types are associated to the C code routes?

ANSWER:

g0/0 and g0/1

Step 3:

what is the operational status of the G0/1 interface?

GigaEthernet0/1 is up, line protocol; is up

What is the media access control (MAC) address of the G0/1 interface?

ANSWER:

How is the Internet address displayed in this command?

ANSWER:

192.168.1.1/24

Step 4:

REFLECTION

- If the G0/1 interface showed administratively down, what interface configuration command would you use to turn the interface up?

ANSWER:

R1(config-if)# no shutdown

- What would happen if you had incorrectly configured interface g0/1 on the router with an IP address of 192.168.1.2?

ANSWER:

PC-A would not be able to ping PC-B. This is because PC-B is on a different network than PC-A which requires the default-gateway router to route these packets. PC-A is configured to use the IP address of 192.168.1.1 for the default-gateway router, but this address is not assigned to any device on the LAN.Any packets that need to be sent to the default-gateway for routing will never reach their destination.