
NETW 203 – SKILLS BASED ASSESSMENT
Name:__________________________

D#_________________

Course:_______________________

1.  Construct the network below in Packet Tracer.  Us the 2621XM router.  You will need to add the NM-2FE2W module in order to have the extra two fast Ethernet ports.
[image: image1.png]2o

Fa0is” ourd
" <X g
° | HOL Fal
60 Hosts 50 Hosts 20 osts 30 Hosts
24
wheho A 2ofhoa 205pXa
it shike P A

PCPT PC-PT PC-PT PC-PT  PCPT ‘pcpT  PC-PT PCPT

pco PCL pcz  PC3 PC4 s pce  PCT




2. You have been given the ip address of: 202.150.47.0/24.  Your task is to create a VLSM addressing system that will support the hosts on each LAN by using the least number of host bits possible.   
a. Use the lowest numbered addresses for the workstations.  
b. Assign the highest useable address to the router interfaces.

c. If your pings are not successful, verify that you have assigned the proper host addresses and re-test for connectivity.
3. When you have everything working properly, copy and paste a successful ping sequence to each of the specified PC destinations from PC0 into a Word document for submission:

a. PC2

b. PC4

c. PC6
4. Open the DOS prompt on PCs 0, 2, 4 and 6 and execute the “ipconfig” command on each of them.  Then, copy and paste the results into your Word document for submission.
5. Use this document as your coversheet and upload the completed lab to the drop box.
NOTE!

THIS ASSIGNMENT MUST BE COMPLETED AND SUMITTED TO THE PROPER DROP BOX BY THE 11:59 PM DEADLINE.

Sample Output
Ping test from PC0 to PC 3

PC>ping 192.168.20.1

Pinging 192.168.20.1 with 32 bytes of data:

Reply from 192.168.20.1: bytes=32 time=18ms TTL=126

Reply from 192.168.20.1: bytes=32 time=15ms TTL=126

Reply from 192.168.20.1: bytes=32 time=19ms TTL=126

Reply from 192.168.20.1: bytes=32 time=22ms TTL=126

Ping statistics for 192.168.20.1:

    Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

    Minimum = 15ms, Maximum = 22ms, Average = 18ms

PC0 IP Addressing Configuration
PC>ipconfig

IP Address......................: 192.168.10.1

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: 192.168.10.254

