| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 | Decimal Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | .128 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | .192 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | .224 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | .240 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | .248 |
| 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | .252 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | .254 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | .255 |
| /25 | /26 | /27 | /28 | /29 | /30 | /31 | /32 |  |

- IP ADDRESS: 192.15.11.3/24

Subnet mask: 255.255.255.0

Network: 192.15.11.0

Broadcast: 192.15.11.255

Host min: 1

Host max: 254

Hosts: 254

Range: 64 (256/4 = 64) including network and broadcast

Subnet mask if it is a 4 equal subnet I need to borough 2 bits (2x2=4 if you look at the table above 2 bits is equal 192 / 26)

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-62 | 63 | 255.255.255.192/26 |
| Subnet 2 | 64 | 65-126 | 127 | 255.255.255.192/26 |
| Subnet 3 | 128 | 129-190 | 191 | 255.255.255.192/26 |
| Subnet 4 | 192 | 193-254 | 255 | 255.255.255.192/26 |

Subnet mask if it is a 8 equal subnet I need to borough 3 bits (2x2x2=8 if you look at the table above 3 bits is equal 224 / 27)

Range: 64 (256/8 = 32) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-30 | 31 | 255.255.255.224/27 |
| Subnet 2 | 32 | 33-62 | 63 | 255.255.255.224/27 |
| Subnet 3 | 64 | 65-94 | 95 | 255.255.255.224/27 |
| Subnet 4 | 96 | 97-126 | 127 | 255.255.255.224/27 |
| Subnet 5 | 128 | 129-158 | 159 | 255.255.255.224/27 |
| Subnet 6 | 160 | 161-190 | 191 | 255.255.255.224/27 |
| Subnet 7 | 192 | 193-222 | 223 | 255.255.255.224/27 |
| Subnet 8 | 224 | 225-254 | 255 | 255.255.255.224/27 |

Range: 16 (256/16=16) including network and broadcast

Subnet mask if it is a 16 equal subnet I need to borough 4 bits (2x2x2x2=16 if you look at the table above 3 bits is equal 240 / 28)

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-14 | 15 | 255.255.255.240/28 |
| Subnet 2 | 16 | 17-20 | 21 | 255.255.255.240/28 |
| Subnet 3 | 22 | 23-36 | 37 | 255.255.255.240/28 |
| Subnet 4 | 38 | 39-52 | 53 | 255.255.255.240/28 |
| Subnet 5 | 54 | 55-68 | 69 | 255.255.255.240/28 |
| Subnet 6 | 70 | 71-84 | 85 | 255.255.255.240/28 |
| Subnet 7 | 86 | 87-100 | 101 | 255.255.255.240/28 |
| Subnet 8 | 102 | 103-116 | 117 | 255.255.255.240/28 |
| Subnet 9 | 118 | 119-132 | 133 | 255.255.255.240/28 |
| Subnet 10 | 134 | 135-148 | 149 | 255.255.255.240/28 |
| Subnet 11 | 150 | 151-164 | 165 | 255.255.255.240/28 |
| Subnet 12 | 166 | 167-182 | 183 | 255.255.255.240/28 |
| Subnet 13 | 182 | 183-196 | 197 | 255.255.255.240/28 |
| Subnet 14 | 198 | 199-212 | 213 | 255.255.255.240/28 |
| Subnet 15 | 214 | 215-218 | 229 | 255.255.255.240/28 |
| Subnet 16 | 230 | 231-254 | 255 | 255.255.255.240/28 |

- 200.34.19.1/25

Network: 200.34.19.0

Broadcast: 200.34.19.127

Host min: 1

Host max: 126

Hosts: 126

Range: 32 (128/4 = 32) – including broadcast and network

Mask: 255.255.255.128

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-3 | 31 | 255.255.255.192/26 |
| Subnet 2 | 32 | 33-62 | 63 | 255.255.255.192/26 |
| Subnet 3 | 64 | 65-94 | 95 | 255.255.255.192/26 |
| Subnet 4 | 96 | 97-127 | 128 | 255.255.255.192/26 |

Range: 16 (128/8=16)

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-14 | 15 | 255.255.255.224/27 |
| Subnet 2 | 16 | 17-30 | 31 | 255.255.255.224/27 |
| Subnet 3 | 32 | 33-46 | 47 | 255.255.255.224/27 |
| Subnet 4 | 48 | 49-62 | 63 | 255.255.255.224/27 |
| Subnet 5 | 64 | 65-78 | 79 | 255.255.255.224/27 |
| Subnet 6 | 80 | 81-94 | 95 | 255.255.255.224/27 |
| Subnet 7 | 96 | 97-110 | 111 | 255.255.255.224/27 |
| Subnet 8 | 112 | 113-127 | 128 | 255.255.255.224/27 |

Range: 8 (128/16=8)

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-6 | 7 | 255.255.255.240/28 |
| Subnet 2 | 8 | 9-14 | 15 | 255.255.255.240/28 |
| Subnet 3 | 16 | 17-22 | 23 | 255.255.255.240/28 |
| Subnet 4 | 24 | 25-30 | 31 | 255.255.255.240/28 |
| Subnet 5 | 32 | 33-38 | 39 | 255.255.255.240/28 |
| Subnet 6 | 40 | 41-46 | 47 | 255.255.255.240/28 |
| Subnet 7 | 48 | 49-54 | 55 | 255.255.255.240/28 |
| Subnet 8 | 56 | 56-62 | 63 | 255.255.255.240/28 |
| Subnet 9 | 64 | 65-70 | 71 | 255.255.255.240/28 |
| Subnet 10 | 72 | 73-78 | 79 | 255.255.255.240/28 |
| Subnet 11 | 80 | 81-86 | 87 | 255.255.255.240/28 |
| Subnet 12 | 88 | 89-94 | 95 | 255.255.255.240/28 |
| Subnet 13 | 96 | 97-102 | 103 | 255.255.255.240/28 |
| Subnet 14 | 104 | 105-110 | 111 | 255.255.255.240/28 |
| Subnet 15 | 112 | 113-118 | 119 | 255.255.255.240/28 |
| Subnet 16 | 120 | 121-127 | 128 | 255.255.255.240/28 |

- IP Address: 10.23.191.45/26

Netmask: 255.255.255.192

Network: 10.23.191.0

Broadcast: 10.23.191.63

Host Min: 1

Host Max: 62

Hosts: 62

Range: 16 (64/4=16) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-14 | 15 | 255.255.255.192/26 |
| Subnet 2 | 16 | 17-30 | 31 | 255.255.255.192/26 |
| Subnet 3 | 32 | 33-46 | 47 | 255.255.255.192/26 |
| Subnet 4 | 48 | 49-62 | 63 | 255.255.255.192/26 |

Range: 8 (64/8=8) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-6 | 7 | 255.255.255.224/27 |
| Subnet 2 | 8 | 9-14 | 15 | 255.255.255.224/27 |
| Subnet 3 | 16 | 17-22 | 23 | 255.255.255.224/27 |
| Subnet 4 | 24 | 25-30 | 31 | 255.255.255.224/27 |
| Subnet 5 | 32 | 33-38 | 39 | 255.255.255.224/27 |
| Subnet 6 | 40 | 38-46 | 47 | 255.255.255.224/27 |
| Subnet 7 | 48 | 49-54 | 55 | 255.255.255.224/27 |
| Subnet 8 | 56 | 57-62 | 63 | 255.255.255.224/27 |

Range: 4 (64/16=4) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-2 | 3 | 255.255.255.240/28 |
| Subnet 2 | 4 | 5-6 | 7 | 255.255.255.240/28 |
| Subnet 3 | 8 | 9-10 | 11 | 255.255.255.240/28 |
| Subnet 4 | 12 | 13-14 | 15 | 255.255.255.240/28 |
| Subnet 5 | 16 | 17-18 | 19 | 255.255.255.240/28 |
| Subnet 6 | 20 | 21-22 | 23 | 255.255.255.240/28 |
| Subnet 7 | 24 | 25-26 | 27 | 255.255.255.240/28 |
| Subnet 8 | 28 | 29-30 | 31 | 255.255.255.240/28 |
| Subnet 9 | 32 | 33-34 | 35 | 255.255.255.240/28 |
| Subnet 10 | 36 | 37-38 | 39 | 255.255.255.240/28 |
| Subnet 11 | 40 | 41-42 | 43 | 255.255.255.240/28 |
| Subnet 12 | 44 | 45-46 | 47 | 255.255.255.240/28 |
| Subnet 13 | 48 | 49-50 | 51 | 255.255.255.240/28 |
| Subnet 14 | 52 | 53-54 | 55 | 255.255.255.240/28 |
| Subnet 15 | 56 | 57-58 | 59 | 255.255.255.240/28 |
| Subnet 16 | 60 | 61-62 | 63 | 255.255.255.240/28 |

- 132.88.19.215/26

Net mask: 255.255.255.192 (borough 2 bits)

Network: 132.88.19.192

Broadcast: 132.88.19.255

Host Min: 193

Host Max: 254

Hosts: 62

Range: 16(64/4=16) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192 | 193-206 | 207 | 255.255.255.192/26 |
| Subnet 2 | 208 | 209-222 | 223 | 255.255.255.192/26 |
| Subnet 3 | 224 | 225-238 | 239 | 255.255.255.192/26 |
| Subnet 4 | 240 | 241-254 | 255 | 255.255.255.192/26 |

Range: 8(64/8=8) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192 | 191-198 | 199 | 255.255.255.224/27 |
| Subnet 2 | 200 | 201-206 | 207 | 255.255.255.224/27 |
| Subnet 3 | 208 | 209-214 | 215 | 255.255.255.224/27 |
| Subnet 4 | 216 | 217-222 | 223 | 255.255.255.224/27 |
| Subnet 5 | 224 | 225-230 | 231 | 255.255.255.224/27 |
| Subnet 6 | 232 | 233-238 | 239 | 255.255.255.224/27 |
| Subnet 7 | 240 | 241-246 | 247 | 255.255.255.224/27 |
| Subnet 8 | 248 | 249-254 | 255 | 255.255.255.224/27 |

Range:  4 (64/16=4) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192 | 193-194 | 195 | 255.255.255.240/28 |
| Subnet 2 | 196 | 197-198 | 199 | 255.255.255.240/28 |
| Subnet 3 | 200 | 201-202 | 203 | 255.255.255.240/28 |
| Subnet 4 | 204 | 205-206 | 207 | 255.255.255.240/28 |
| Subnet 5 | 208 | 209-210 | 211 | 255.255.255.240/28 |
| Subnet 6 | 212 | 213-214 | 215 | 255.255.255.240/28 |
| Subnet 7 | 216 | 217-218 | 219 | 255.255.255.240/28 |
| Subnet 8 | 220 | 221-222 | 223 | 255.255.255.240/28 |
| Subnet 9 | 224 | 225-226 | 227 | 255.255.255.240/28 |
| Subnet 10 | 228 | 229-230 | 231 | 255.255.255.240/28 |
| Subnet 11 | 232 | 233-234 | 235 | 255.255.255.240/28 |
| Subnet 12 | 236 | 237-238 | 239 | 255.255.255.240/28 |
| Subnet 13 | 240 | 241-242 | 243 | 255.255.255.240/28 |
| Subnet 14 | 244 | 245-246 | 247 | 255.255.255.240/28 |
| Subnet 15 | 248 | 249-250 | 251 | 255.255.255.240/28 |
| Subnet 16 | 252 | 253-254 | 255 | 255.255.255.240/28 |

- 132.88.19.215/27

Network: 132.88.19.192

Broadcast: 132.88.19.223

Host Min: 193

Host Max: 222

Hosts: 30

Range: 8(32/4=8) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192 | 193-198 | 199 | 255.255.255.192/26 |
| Subnet 2 | 200 | 201-206 | 207 | 255.255.255.192/26 |
| Subnet 3 | 208 | 208-214 | 215 | 255.255.255.192/26 |
| Subnet 4 | 216 | 217-222 | 223 | 255.255.255.192/26 |

Range: 4(32/8=4) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 192 | 193-194 | 195 | 255.255.255.224/27 |
| Subnet 2 | 196 | 197-198 | 199 | 255.255.255.224/27 |
| Subnet 3 | 200 | 201-203 | 203 | 255.255.255.224/27 |
| Subnet 4 | 204 | 205-206 | 207 | 255.255.255.224/27 |
| Subnet 5 | 208 | 209-210 | 211 | 255.255.255.224/27 |
| Subnet 6 | 212 | 213-214 | 215 | 255.255.255.224/27 |
| Subnet 7 | 216 | 217-218 | 219 | 255.255.255.224/27 |
| Subnet 8 | 220 | 221-222 | 223 | 255.255.255.224/27 |

Range: 2(32/16=2) not possible I need minimum 4 for each subnet

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  | 255.255.255.240/28 |
| Subnet 2 |  |  |  | 255.255.255.240/28 |
| Subnet 3 |  |  |  | 255.255.255.240/28 |
| Subnet 4 |  |  |  | 255.255.255.240/28 |
| Subnet 5 |  |  |  | 255.255.255.240/28 |
| Subnet 6 |  |  |  | 255.255.255.240/28 |
| Subnet 7 |  |  |  | 255.255.255.240/28 |
| Subnet 8 |  |  |  | 255.255.255.240/28 |
| Subnet 9 |  |  |  | 255.255.255.240/28 |
| Subnet 10 |  |  |  | 255.255.255.240/28 |
| Subnet 11 |  |  |  | 255.255.255.240/28 |
| Subnet 12 |  |  |  | 255.255.255.240/28 |
| Subnet 13 |  |  |  | 255.255.255.240/28 |
| Subnet 14 |  |  |  | 255.255.255.240/28 |
| Subnet 15 |  |  |  | 255.255.255.240/28 |
| Subnet 16 |  |  |  | 255.255.255.240/28 |

- 132.88.19.215

Netmask: 255.255.255.192 (26)

Network: 132.88.19.192

Broadcast: 132.88.19.255

Host Min: 193

Host Max: 254

Hosts: 62

SAME as NUMBER 4

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  | 255.255.255.192/26 |
| Subnet 2 |  |  |  | 255.255.255.192/26 |
| Subnet 3 |  |  |  | 255.255.255.192/26 |
| Subnet 4 |  |  |  | 255.255.255.192/26 |

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  | 255.255.255.224/27 |
| Subnet 2 |  |  |  | 255.255.255.224/27 |
| Subnet 3 |  |  |  | 255.255.255.224/27 |
| Subnet 4 |  |  |  | 255.255.255.224/27 |
| Subnet 5 |  |  |  | 255.255.255.224/27 |
| Subnet 6 |  |  |  | 255.255.255.224/27 |
| Subnet 7 |  |  |  | 255.255.255.224/27 |
| Subnet 8 |  |  |  | 255.255.255.224/27 |

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  | 255.255.255.240/28 |
| Subnet 2 |  |  |  | 255.255.255.240/28 |
| Subnet 3 |  |  |  | 255.255.255.240/28 |
| Subnet 4 |  |  |  | 255.255.255.240/28 |
| Subnet 5 |  |  |  | 255.255.255.240/28 |
| Subnet 6 |  |  |  | 255.255.255.240/28 |
| Subnet 7 |  |  |  | 255.255.255.240/28 |
| Subnet 8 |  |  |  | 255.255.255.240/28 |
| Subnet 9 |  |  |  | 255.255.255.240/28 |
| Subnet 10 |  |  |  | 255.255.255.240/28 |
| Subnet 11 |  |  |  | 255.255.255.240/28 |
| Subnet 12 |  |  |  | 255.255.255.240/28 |
| Subnet 13 |  |  |  | 255.255.255.240/28 |
| Subnet 14 |  |  |  | 255.255.255.240/28 |
| Subnet 15 |  |  |  | 255.255.255.240/28 |
| Subnet 16 |  |  |  | 255.255.255.240/28 |

- 192.168.1.111

Netmask: 255.255.224(27)

Network:  192.168.1.96

Broadcast: 192.168.1.127

Host Min: 97

Host Max: 126

Hosts: 30

Range: 8(32/4=8) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 96 | 97-102 | 103 | 255.255.255.192/26 |
| Subnet 2 | 104 | 105-110 | 111 | 255.255.255.192/26 |
| Subnet 3 | 112 | 111-118 | 119 | 255.255.255.192/26 |
| Subnet 4 | 120 | 121-126 | 127 | 255.255.255.192/26 |

Range: 4(32/8=4) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 96 | 97-98 | 99 | 255.255.255.224/27 |
| Subnet 2 | 100 | 101-102 | 103 | 255.255.255.224/27 |
| Subnet 3 | 104 | 105-106 | 107 | 255.255.255.224/27 |
| Subnet 4 | 108 | 109-110 | 111 | 255.255.255.224/27 |
| Subnet 5 | 112 | 113-114 | 115 | 255.255.255.224/27 |
| Subnet 6 | 116 | 117-118 | 119 | 255.255.255.224/27 |
| Subnet 7 | 120 | 121-122 | 123 | 255.255.255.224/27 |
| Subnet 8 | 124 | 125-126 | 127 | 255.255.255.224/27 |

Range: not possible (32/16=2) it needs minimum 4 subnets

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  |  |
| Subnet 2 |  |  |  |  |
| Subnet 3 |  |  |  |  |
| Subnet 4 |  |  |  |  |
| Subnet 5 |  |  |  |  |
| Subnet 6 |  |  |  |  |
| Subnet 7 |  |  |  |  |
| Subnet 8 |  |  |  |  |
| Subnet 9 |  |  |  |  |
| Subnet 10 |  |  |  |  |
| Subnet 11 |  |  |  |  |
| Subnet 12 |  |  |  |  |
| Subnet 13 |  |  |  |  |
| Subnet 14 |  |  |  |  |
| Subnet 15 |  |  |  |  |
| Subnet 16 |  |  |  |  |

- 145.11.56.56

Netmask: 255.255.0

Network:  145.11.56.0

Broadcast: 192.168.1.255

Host Min: 1

Host Max: 254

Hosts: 254

Range: 256/4=64 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-62 | 63 | 255.255.255.192/26 |
| Subnet 2 | 64 | 65-126 | 127 | 255.255.255.192/26 |
| Subnet 3 | 128 | 129-190 | 191 | 255.255.255.192/26 |
| Subnet 4 | 192 | 193-254 | 255 | 255.255.255.192/26 |

Range: 256/8=32 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-31 | 33 | 255.255.255.224/27 |
| Subnet 2 | 32 | 33-62 | 63 | 255.255.255.224/27 |
| Subnet 3 | 64 | 65-94 | 95 | 255.255.255.224/27 |
| Subnet 4 | 96 | 97-126 | 127 | 255.255.255.224/27 |
| Subnet 5 | 128 | 129-158 | 159 | 255.255.255.224/27 |
| Subnet 6 | 160 | 161-190 | 191 | 255.255.255.224/27 |
| Subnet 7 | 192 | 193-222 | 223 | 255.255.255.224/27 |
| Subnet 8 | 224 | 225-254 | 255 | 255.255.255.224/27 |

Range: 256/16=16 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 0 | 1-14 | 15 | 255.255.255.240/28 |
| Subnet 2 | 16 | 17-30 | 31 | 255.255.255.240/28 |
| Subnet 3 | 32 | 33-46 | 47 | 255.255.255.240/28 |
| Subnet 4 | 48 | 49-62 | 63 | 255.255.255.240/28 |
| Subnet 5 | 64 | 65-78 | 79 | 255.255.255.240/28 |
| Subnet 6 | 80 | 81-94 | 95 | 255.255.255.240/28 |
| Subnet 7 | 96 | 97-110 | 111 | 255.255.255.240/28 |
| Subnet 8 | 112 | 113-126 | 127 | 255.255.255.240/28 |
| Subnet 9 | 128 | 129-142 | 143 | 255.255.255.240/28 |
| Subnet 10 | 144 | 145-157 | 159 | 255.255.255.240/28 |
| Subnet 11 | 160 | 161-174 | 175 | 255.255.255.240/28 |
| Subnet 12 | 172 | 177-190 | 191 | 255.255.255.240/28 |
| Subnet 13 | 192 | 196-206 | 207 | 255.255.255.240/28 |
| Subnet 14 | 208 | 209-222 | 223 | 255.255.255.240/28 |
| Subnet 15 | 224 | 225-238 | 239 | 255.255.255.240/28 |
| Subnet 16 | 240 | 241-254 | 255 | 255.255.255.240/28 |

- 45.25.45.45 Subnet Mask 255.255.255.240

Network = 45.25.45.32

Broadcast = 45.25.45.47

Host Min: 33

Host Max: 46

Hosts: 14

Range: 4 (16/4=4) including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 32 | 33-34 | 35 | 255.255.255.192/26 |
| Subnet 2 | 36 | 37-38 | 39 | 255.255.255.192/26 |
| Subnet 3 | 40 | 41-42 | 43 | 255.255.255.192/26 |
| Subnet 4 | 44 | 45-47 | 48 | 255.255.255.192/26 |

Not possible needs minimum 4 for each subnet

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  |  |
| Subnet 2 |  |  |  |  |
| Subnet 3 |  |  |  |  |
| Subnet 4 |  |  |  |  |
| Subnet 5 |  |  |  |  |
| Subnet 6 |  |  |  |  |
| Subnet 7 |  |  |  |  |
| Subnet 8 |  |  |  |  |

Not Possible needs minimum 4 for each subnet

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 |  |  |  |  |
| Subnet 2 |  |  |  |  |
| Subnet 3 |  |  |  |  |
| Subnet 4 |  |  |  |  |
| Subnet 5 |  |  |  |  |
| Subnet 6 |  |  |  |  |
| Subnet 7 |  |  |  |  |
| Subnet 8 |  |  |  |  |
| Subnet 9 |  |  |  |  |
| Subnet 10 |  |  |  |  |
| Subnet 11 |  |  |  |  |
| Subnet 12 |  |  |  |  |
| Subnet 13 |  |  |  |  |
| Subnet 14 |  |  |  |  |
| Subnet 15 |  |  |  |  |
| Subnet 16 |  |  |  |  |

- 198.198.198.198 Subnet Mask 255.255.255.128 (25)

Network = 198.198.198.128

Broadcast = 198.198.198.255

Host Min: 129

Host Max: 254

Host 126

Range: 128/4=32 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 128 | 129-158 | 159 | 255.255.255.192/26 |
| Subnet 2 | 160 | 161-190 | 191 | 255.255.255.192/26 |
| Subnet 3 | 192 | 193-222 | 223 | 255.255.255.192/26 |
| Subnet 4 | 224 | 225-254 | 255 | 255.255.255.192/26 |

Range: 128/8=16 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 128 | 129-142 | 143 | 255.255.255.224/27 |
| Subnet 2 | 144 | 145-158 | 159 | 255.255.255.224/27 |
| Subnet 3 | 160 | 161-170 | 171 | 255.255.255.224/27 |
| Subnet 4 | 172 | 173-190 | 191 | 255.255.255.224/27 |
| Subnet 5 | 192 | 193-206 | 207 | 255.255.255.224/27 |
| Subnet 6 | 208 | 209-222 | 223 | 255.255.255.224/27 |
| Subnet 7 | 224 | 225-238 | 239 | 255.255.255.224/27 |
| Subnet 8 | 240 | 241-254 | 255 | 255.255.255.224/27 |

Range: 128/16=8 including network and broadcast

| Name | Subnet Address | Host Range | Broadcast | Mask |
| --- | --- | --- | --- | --- |
| Subnet 1 | 128 | 129-134 | 135 | 255.255.255.240/28 |
| Subnet 2 | 136 | 137-142 | 143 | 255.255.255.240/28 |
| Subnet 3 | 144 | 145-150 | 151 | 255.255.255.240/28 |
| Subnet 4 | 152 | 153-158 | 159 | 255.255.255.240/28 |
| Subnet 5 | 160 | 161-166 | 167 | 255.255.255.240/28 |
| Subnet 6 | 168 | 169-174 | 175 | 255.255.255.240/28 |
| Subnet 7 | 176 | 177-182 | 183 | 255.255.255.240/28 |
| Subnet 8 | 184 | 185-190 | 191 | 255.255.255.240/28 |
| Subnet 9 | 192 | 193-198 | 199 | 255.255.255.240/28 |
| Subnet 10 | 200 | 201-206 | 207 | 255.255.255.240/28 |
| Subnet 11 | 208 | 209-214 | 215 | 255.255.255.240/28 |
| Subnet 12 | 216 | 217-222 | 223 | 255.255.255.240/28 |
| Subnet 13 | 224 | 225-230 | 231 | 255.255.255.240/28 |
| Subnet 14 | 232 | 233-238 | 239 | 255.255.255.240/28 |
| Subnet 15 | 240 | 241-246 | 247 | 255.255.255.240/28 |
| Subnet 16 | 248 | 249-254 | 255 | 255.255.255.240/28 |